ARM ISA

- Arithmetic / Data Processing

  - ADD - Adds two operands and writes result to destination register.

  - SUB - Subtracts second operand from first and writes result.

  - MOV - Copies a value (register or immediate) into a register.

  - CMP - Performs subtraction only to set condition flags, does NOT write result.

  - LSL - Logical shift left (used to multiply index by 4 for array access).

    - Ex - MOV r3, r3, LSL #2             [Mov is used as the instruction for LSL]

- Memory Access (Single Register)

  - LDR - Loads a 32-bit word from memory into a register.

  - STR - Stores a 32-bit register value into memory.

- Memory Access (Multiple Registers)

  - LDM - Load multiple registers from memory starting at base address.

  - STM - Store multiple registers to memory starting at base address.

  - LDMIA - Load multiple registers, increment address after each load.

  - STMIA - Store multiple registers, increment address after each store.

- Stack Operations (Aliases)

  - PUSH - Stores multiple registers onto the stack (alias for STMDB).

  - POP - Loads multiple registers from the stack (alias for LDMIA).

- Control Flow

  - B - Unconditional branch to a label.

    - What hardware does:

      - Calculate branch target address

      - Replace PC with that address

      - No flags involved.

  - BLE - Branch if Less Than or Equal (based on flags).

    - BLE condition = (Z == 1) OR (N != V)

      - Z == 1 : for equal to

      - N != V :  No overflow should be there, only then the true MSB is seen so V == 0. True MSB is 1 means its negative that means A<B. (V == 0 is not equal to N == 1)

  - BGE - Branch if Greater Than or Equal (based on flags).

    - BGE condition = (N == V)

      - No overflow but if not negative then it means A>B. (V == 0 is equal to N == 0)

  - BX - Branch to address stored in register (used for return).

This instruction performs a branch by copying the contents of a general register, Rn, into the program counter, PC

    - Hardware must:

      - Read value of register (lr)

      - Load it into PC

      - No offset.

      - No condition.

- Special Register Names:

| Register | Alias | Meaning |
| --- | --- | --- |
| r13 | sp | Stack Pointer |
| r14 | lr | Link Register |
| r15 | pc | Program Counter |
| r11 | fp | Frame Pointer (by convention) |